The following are requirements for the Flight Control Computer software of a UAV. Please generate Simulink Models with Design Verifier harnesses such that the requirements can be formally verified. Please also generate safety properties in terms of LTL, CTL, propositional, predicate, or other logic that can be verified using SLDV.

The Control Computer shall use the barometric altitude read from the sensor as the actual altitude of the UAV.

The Control Computer shall execute the altitude control loop to set the pitch angle based on the difference between the reference altitude set by the pilot from the ground and the actual altitude of the UAV.

The Control Computer shall use the indicative airspeed read from the air data probes as the actual airspeed of the UAV.

The Control Computer shall execute the speed control loop to set the engine throttle based on the difference between reference airspeed and actual airspeed.

If the reference altitude is more than 100 meters higher than the actual altitude and remains so for more than 1 second, the Control Computer shall enter Climb mode. In the Climb mode, the Control Computer shall pitch up 10 degrees and set the engine throttle to 100% to gain altitude. Suppose the actual altitude equals or exceeds the reference altitude and remains so for more than 1 second. In that case, the Control Computer shall exit the Climb mode and set the pitch angle and engine throttle based on the altitude control loop and speed control loop, respectively.

At any time, if the actual airspeed is below the stall speed and remains so for more than 1 second, the Control Computer shall enter Stall Recovery mode. In Stall Recovery mode, the Control Computer shall set the engine throttle to 100% and Pitch angle to 0 degrees or as computed by the altitude control loop, whichever is lower. The Control Computer shall exit the Stall Recovery mode when the actual airspeed is more than (stall speed + 5 meters per second) and remains so continuously for 1 second.

The Stall Recovery mode shall have higher priority than the Climb mode.

The rise in the engine throttle shall be limited to 10% per second at all times.

The engine throttle shall be within the limits of a minimum of 40% (corresponding to idle throttle) and a maximum of 100%. Hence, once the Control Computer enters either the Climb or the Stall Recovery mode, the engine throttle shall become 100% within a maximum of 6 seconds.

Assume that the controller runs every 10 milliseconds.

OUTPUT OF GEMINI

Based on the requirements provided, here is a design for a Simulink model with a Design Verifier harness, along with safety properties expressed in logical formalisms. The Simulink models can be created to represent the flight control logic, and the Design Verifier harness can be used to formally verify the system's behavior against the specified properties.

Simulink Model Structure

The Simulink model would be built with several key components to reflect the UAV's flight control system.

Inputs

The model would have the following inputs, representing sensor readings and pilot commands:

barometric_altitude: The actual altitude of the UAV (m).

indicated_airspeed: The actual airspeed of the UAV (m/s).

reference_altitude: The altitude set by the pilot (m).

reference_airspeed: The airspeed set by the pilot (m/s).

stall_speed: A constant value representing the UAV's stall speed (m/s).

Outputs

The model would generate the following control outputs:

pitch_angle_command: The commanded pitch angle (degrees).

engine_throttle_command: The commanded engine throttle percentage (%).

Subsystems

The main control logic would be structured into several interconnected subsystems:

Altitude Control Loop: This subsystem takes the reference_altitude and barometric_altitude as inputs. It calculates a nominal pitch angle command based on their difference. This is a proportional-integral-derivative (PID) controller or a similar control law.

Speed Control Loop: This subsystem takes the reference_airspeed and indicated_airspeed as inputs. It calculates a nominal engine throttle command based on their difference. This is also likely a PID controller.

Mode Logic (Stateflow Chart): This is the core of the system and would be implemented using a Stateflow chart. It would handle the transitions between different operational modes and enforce the priority and time-based conditions. The states would include:

Normal: The default mode where the pitch and throttle are determined by the altitude and speed control loops.

Climb: A mode entered when a significant altitude increase is required.

Stall_Recovery: The highest-priority mode, entered when the airspeed drops below the stall speed.

The Stateflow chart would manage the mode transitions based on timed events. For example, it would use internal timers to track how long a condition (like reference_altitude > barometric_altitude + 100) has been true.

Throttle Limiter: This subsystem would take the raw engine throttle command and enforce the rate of change and the absolute limits. It would ensure that the throttle does not increase by more than 10% per second and remains between 40% and 100%. This can be implemented using a Unit Delay block and saturation blocks.

The Simulink model would be built using blocks like Sum, Gain, PID Controller, Relational Operator, Switch, and Stateflow. The Stateflow chart is critical for implementing the sequential and prioritized logic.

Formal Verification Properties

These properties, written in logical form, can be used with the Simulink Design Verifier to formally verify the system's behavior. Design Verifier can prove whether these properties are always true, false, or if there's a counterexample.

Safety Properties (LTL/CTL-like)

Throttle Rate Limit: The throttle command shall never increase by more than 10% per second.

Formal Property:

G(engine_throttle_command(t)−engine_throttle_command(t−1)≤10)

(Where t is the current time step and t-1 is the previous time step, assuming a 1-second interval or scaled accordingly for the 10ms sampling time.)

Throttle Range: The engine throttle command shall always be between 40% and 100%.

Formal Property:

G(40≤engine_throttle_command≤100)

Climb Mode Entry Condition: If the reference altitude is more than 100 meters higher than the actual altitude and remains so for more than 1 second, the system shall eventually enter Climb mode.

Formal Property:

G((reference_altitude−barometric_altitude>100∧X(reference_altitude−barometric_altitude>100∧…100 times))⇒F(mode=Climb))

(The X operator here represents the "next" state in discrete time, with 100 repetitions for the 1-second delay at a 10ms sampling rate).

Climb Mode Exit Condition: If the system is in Climb mode and the actual altitude equals or exceeds the reference altitude and remains so for more than 1 second, the system shall eventually exit Climb mode.

Formal Property:

G((mode=Climb∧barometric_altitude≥reference_altitude∧X(barometric_altitude≥reference_altitude∧…100 times))⇒F(mode=Climb))

Stall Recovery Priority: If the stall condition is met, the system shall always enter Stall Recovery mode, regardless of other conditions. The Stall Recovery mode shall not be exited until the airspeed is sufficiently high.

Formal Property:

$$\mathbf{G}((\text{indicated\_airspeed} < \text{stall\_speed} \land \mathbf{X}(\dots \text{100 times})) \Rightarrow \mathbf{F}(\text{mode} = \text{Stall\_Recovery})) $$ $$ \mathbf{G}((\text{mode} = \text{Stall\_Recovery} \land \text{indicated\_airspeed} \leq \text{stall\_speed} + 5) \Rightarrow \mathbf{X}(\text{mode} = \text{Stall\_Recovery}))$$

Stall Recovery Throttle and Pitch: When in Stall Recovery mode, the throttle shall be 100%, and the pitch shall be the minimum of 0 degrees and the altitude control loop's output.

Formal Property:

G(mode=Stall_Recovery⇒(engine_throttle_command=100∧pitch_angle_command=min(0,pitch_from_altitude_loop)))

(This requires exposing the raw output of the altitude loop to the verifier.)

Throttle Ramp: When entering a mode that commands 100% throttle, the throttle must reach 100% within a maximum of 6 seconds.

Formal Property:

G((enter_Climb∨enter_Stall_Recovery)⇒F600​(engine_throttle_command=100))

(The F_600 operator means "eventually, within 600 time steps," which corresponds to 6 seconds at a 10ms sampling rate.)

OUTPUT OF MISTRAL

I'm unable to directly generate Simulink Models or Design Verifier harnesses as I don't have access to the necessary tools like MATLAB or Simulink. However, I can certainly guide you on how to approach this task and provide you with the logical framework and safety properties you need.

Approach to Generate Simulink Models

- System Architecture:

  - Sensors: Barometric altitude sensor and airspeed sensor.

  - Control Loops: Altitude control loop and speed control loop.

  - Modes: Normal, Climb, and Stall Recovery modes.

  - Actuators: Pitch angle and engine throttle.

- Subsystems:

  - Sensor Inputs: Blocks to read barometric altitude and airspeed.

  - Control Loops: PID controllers for altitude and speed.

  - Mode Logic: Stateflow chart or logical blocks to handle mode transitions.

  - Actuator Outputs: Blocks to set pitch angle and engine throttle.

Design Verifier Harnesses

- Requirements-Based Testing:

  - Create test harnesses for each requirement.

  - Use Design Verifier to generate test cases and formally verify the model.

- Safety Properties:

  - Define safety properties using temporal logic (LTL, CTL) and verify them using Simulink Design Verifier (SLDV).

Safety Properties

- Altitude Control:

  - LTL: G (|reference_altitude - actual_altitude| < threshold -> F (|reference_altitude - actual_altitude| < threshold))

  - CTL: AG (|reference_altitude - actual_altitude| < threshold -> AF (|reference_altitude - actual_altitude| < threshold))

- Speed Control:

  - LTL: G (actual_airspeed > stall_speed -> F (actual_airspeed > stall_speed))

  - CTL: AG (actual_airspeed > stall_speed -> AF (actual_airspeed > stall_speed))

- Climb Mode:

  - LTL: G ((reference_altitude - actual_altitude > 100) -> F (mode = Climb))

  - CTL: AG ((reference_altitude - actual_altitude > 100) -> AF (mode = Climb))

- Stall Recovery Mode:

  - LTL: G (actual_airspeed < stall_speed -> F (mode = Stall_Recovery))

  - CTL: AG (actual_airspeed < stall_speed -> AF (mode = Stall_Recovery))

- Mode Priority:

  - Propositional Logic: Stall_Recovery_mode -> !Climb_mode

- Throttle Limits:

  - Predicate Logic: 40 <= engine_throttle <= 100

  - LTL: G (engine_throttle <= 100 & engine_throttle >= 40)

- Throttle Rate Limit:

  - LTL: G (engine_throttle(t+1) - engine_throttle(t) <= 10%)

Steps to Implement in Simulink

- Create the Model:

  - Open Simulink and create a new model.

  - Add blocks for sensor inputs, control loops, mode logic, and actuator outputs.

- Define Requirements:

  - Use Simulink Requirements to document the requirements.

  - Link requirements to the corresponding blocks in the model.

- Create Test Harnesses:

  - Use Simulink Design Verifier to create test harnesses.

  - Define input ranges and constraints for the test harnesses.

- Formal Verification:

  - Use SLDV to generate test cases and formally verify the model.

  - Define safety properties using temporal logic and verify them.

Example Simulink Model Structure

- Sensor Inputs:

  - Barometric Altitude Sensor: Block to read altitude.

  - Airspeed Sensor: Block to read airspeed.

- Control Loops:

  - Altitude Control Loop: PID controller to set pitch angle.

  - Speed Control Loop: PID controller to set engine throttle.

- Mode Logic:

  - Stateflow Chart: To handle mode transitions (Normal, Climb, Stall Recovery).

  - Logical Blocks: To implement mode priority and conditions.

- Actuator Outputs:

  - Pitch Angle: Block to set pitch angle.

  - Engine Throttle: Block to set engine throttle with rate limits.

Example Safety Properties in LTL

- Altitude Control:

- G (|reference_altitude - actual_altitude| < threshold -> F (|reference_altitude - actual_altitude| < threshold))

- Speed Control:

- G (actual_airspeed > stall_speed -> F (actual_airspeed > stall_speed))

- Climb Mode:

- G ((reference_altitude - actual_altitude > 100) -> F (mode = Climb))

- Stall Recovery Mode:

- G (actual_airspeed < stall_speed -> F (mode = Stall_Recovery))

- Throttle Limits:

- G (engine_throttle <= 100 & engine_throttle >= 40)

- Throttle Rate Limit:

- G (engine_throttle(t+1) - engine_throttle(t) <= 10%)

By following these steps and using the provided safety properties, you can create a Simulink model that meets the requirements and can be formally verified using Simulink Design Verifier.

OUTPUT OF CHATGPT

To generate Simulink models and the necessary verification harnesses for the provided UAV Flight Control Computer requirements, we can break down the system into several components and work through the necessary formal verification process.

Breakdown of the Requirements

- Barometric Altitude and Altitude Control Loop:

  - The barometric altitude is used as the actual altitude of the UAV.

  - Altitude control loop adjusts pitch angle based on the difference between the reference altitude set by the pilot and the actual altitude of the UAV.

- Airspeed and Speed Control Loop:

  - The indicative airspeed from the air data probes is used as the actual airspeed.

  - Speed control loop adjusts engine throttle based on the difference between reference airspeed and actual airspeed.

- Climb Mode:

  - If the reference altitude is 100 meters above the actual altitude and remains for more than 1 second, the control system enters Climb mode.

  - In Climb mode, the control system pitches up 10° and sets engine throttle to 100% to gain altitude.

- Exiting Climb Mode:

  - When the actual altitude equals or exceeds the reference altitude for more than 1 second, the control system exits Climb mode.

- Stall Recovery Mode:

  - If the actual airspeed falls below the stall speed and remains so for 1 second, the system enters Stall Recovery mode.

  - In Stall Recovery mode, the control system sets engine throttle to 100% and pitch angle to 0 degrees or as computed by the altitude control loop (whichever is lower).

  - The system exits Stall Recovery mode when the actual airspeed exceeds (stall speed + 5 meters per second) for 1 second.

  - Stall Recovery mode has higher priority than Climb mode.

- Throttle Limitation:

  - The rise in engine throttle is limited to 10% per second.

  - The throttle must stay within the range [40%, 100%].

  - The system enters Climb or Stall Recovery mode and the engine throttle must reach 100% within 6 seconds.

- Controller Timing:

  - The controller runs every 10 ms (0.01 s).

Simulink Model Structure

- Inputs:

  - Reference altitude (set by the pilot)

  - Actual barometric altitude

  - Reference airspeed

  - Actual airspeed

  - Stall speed

- Control Loops:

  - Altitude Control Loop: Computes the required pitch angle based on the altitude error (reference - actual altitude).

  - Speed Control Loop: Computes the required engine throttle based on the airspeed error (reference airspeed - actual airspeed).

- Modes:

  - Climb Mode: Active if altitude error is greater than 100 meters for 1 second.

  - Stall Recovery Mode: Active if the airspeed is below stall speed for 1 second.

- Throttle Limits:

  - Throttle rise is limited to 10% per second.

  - Throttle bounds are limited to [40%, 100%].

Safety Properties (Formal Verification)

To verify the system behavior formally, we define the following Safety Properties using Linear Temporal Logic (LTL) and Computation Tree Logic (CTL):

1. Climb Mode Activation

LTL property to ensure the system enters Climb mode when the altitude error is more than 100 meters and lasts for at least 1 second:

ϕClimb: (altitude error > 100 meters) ⟶ G(Climb mode activated after 1 second)\phi_{\text{Climb}}: \, \text{(altitude error > 100 meters)} \, \longrightarrow \, \text{G(Climb mode activated after 1 second)}ϕClimb​:(altitude error > 100 meters)⟶G(Climb mode activated after 1 second)

2. Climb Mode Exit

LTL property to ensure the system exits Climb mode when the actual altitude reaches or exceeds the reference altitude for at least 1 second:

ϕExitClimb: (actual altitude >= reference altitude) ⟶ G(Climb mode deactivated after 1 second)\phi_{\text{ExitClimb}}: \, \text{(actual altitude >= reference altitude)} \, \longrightarrow \, \text{G(Climb mode deactivated after 1 second)}ϕExitClimb​:(actual altitude >= reference altitude)⟶G(Climb mode deactivated after 1 second)

3. Stall Recovery Mode Activation

LTL property to ensure the system enters Stall Recovery mode if the airspeed falls below the stall speed for 1 second:

ϕStallRecovery: (airspeed < stall speed) ⟶ G(Stall Recovery mode activated after 1 second)\phi_{\text{StallRecovery}}: \, \text{(airspeed < stall speed)} \, \longrightarrow \, \text{G(Stall Recovery mode activated after 1 second)}ϕStallRecovery​:(airspeed < stall speed)⟶G(Stall Recovery mode activated after 1 second)

4. Stall Recovery Mode Exit

LTL property to ensure the system exits Stall Recovery mode when airspeed exceeds (stall speed + 5 m/s) for 1 second:

ϕExitStall: (airspeed > stall speed + 5) ⟶ G(Stall Recovery mode deactivated after 1 second)\phi_{\text{ExitStall}}: \, \text{(airspeed > stall speed + 5)} \, \longrightarrow \, \text{G(Stall Recovery mode deactivated after 1 second)}ϕExitStall​:(airspeed > stall speed + 5)⟶G(Stall Recovery mode deactivated after 1 second)

5. Throttle Rate Limitation

Propositional property to ensure the throttle does not increase by more than 10% per second:

\phi_{\text{ThrottleRate}}: \, \text{(throttle rise rate < 10% per second)}

6. Throttle Boundaries

Predicate property to ensure that the throttle remains within the limits of 40% and 100%:

\phi_{\text{ThrottleLimits}}: \, \text{(throttle >= 40%) \, and \, (throttle <= 100%)}

7. Climb and Stall Recovery Mode Priority

CTL property to enforce that Stall Recovery mode has higher priority over Climb mode:

ϕPriority: AG((Stall Recovery mode activated)   \longrightarrow  (Climb mode not active))\phi_{\text{Priority}}: \, \text{AG((Stall Recovery mode activated) \, \longrightarrow \, \text{(Climb mode not active))}}ϕPriority​:AG((Stall Recovery mode activated)  \longrightarrow (Climb mode not active))

Simulink Model Elements

- Altitude Control Loop:

  - A simple proportional-integral (PI) controller or PID controller.

  - Error between reference and actual altitude fed to the controller to generate the pitch angle.

- Speed Control Loop:

  - A PI or PID controller.

  - Error between reference airspeed and actual airspeed fed to the controller to generate the throttle.

- Climb Mode Logic:

  - A state machine or if-else logic that activates when the altitude error is above 100 meters for more than 1 second.

- Stall Recovery Mode Logic:

  - A state machine or if-else logic that activates when airspeed is below stall speed for more than 1 second.

- Throttle Rate Limiter:

  - A rate limiter block to ensure throttle does not increase more than 10% per second.

- Safety Constraints:

  - Boundaries for throttle (min 40%, max 100%).

  - Constraints for throttle rise rate.

Verification with Simulink Design Verifier (SLDV)

Once the Simulink model is built, the Simulink Design Verifier (SLDV) can be used to check for:

- Property Violations: Use the LTL, CTL, and propositional properties defined earlier to automatically check if the system violates any of the properties.

- Test Generation: SLDV can also generate test cases to cover different operational scenarios.

- Coverage Analysis: Ensure that the control loops and logic satisfy the specified requirements under all conditions.

Next Steps:

- Model Construction: Build a Simulink model that represents the control loops, mode activation logic, and throttle constraints.

- Formal Verification: Use Simulink Design Verifier to verify the safety properties.

- Test Cases: Automatically generate test cases to ensure that the system operates correctly under all specified conditions.

Would you like me to help you start with a specific part of the model or provide guidance on using Simulink Design Verifier for formal verification?