Requirements to model in Simulink+Stateflow

External Inputs:

- Reference altitude (meters)

- Actual altitude (meters)

- Reference speed (m/s)

- Actual speed (m/s)

Constants:

- ALT_THRESHOLD = 100 meters

- STALL_SPEED = 20 m/s

- PERSISTENCT = 100 (equivalent to 1 second with 10 msec execution rate)

- PITCH_CMD_LIMIT = 20 degrees

- THROTTLE_UP_RATE_LIMIT = 10% per second (0.1 per frame)

- GAIN = 0.1

- CLIMB_PITCH_CMD = 5 degrees

Outputs:

- Pitch command (in degrees)

- Engine Throttle (in %)

Requirements:

- The Controller is in Normal mode at start up. In normal mode, it produces outputs as below:

  - Pitch Cmd = (Reference Alt – Actual Alt)*GAIN (limited to +/-  PITCH_CMD_LIMIT deg)

  - Engine Throttle = (Reference speed – Actual Speed)*GAIN (limited to 0-100%. Throttle increase is rate limited to THROTTLE_UP_RATE_LIMIT, decrease is not rate limited)

- While in Normal mode, if the Actual Alt falls below Reference Alt by more than ALT_THRESHOLD continuously for PERSISTENT frames or more, the Controller will go to CLIMB mode.

- While in Climb mode, the Pitch command is set to CLIMB_PITCH_CMD deg and Throttle is set to 100% (with rate limiting)

- While in Climb mode, if the Actual Alt is more than Reference Alt continuously for PERSISTENT frames or more, the Controller goes to Normal mode.

- While in either Normal or Climb mode, if the Actual speed reduces below STALL_SPEED continuously for PERSISTENT frames or more, the Controller goes to StallRecovery mode. In StallRecovery mode, the Pitch Cmd is set to 0deg or as computed in 1.a. above, whichever is lesser. Engine Throttle is set to 100% (with rate limiting).

- While in StallRecovery mode, if the Actual Speed is higher than STALL_SPEED continuously for PERSISTENT frames or more, the Controller shall go to Normal mode.

- If the conditions of Climb and StallRecovery modes are set simultaneously, the StallRecovery shall have higher priority.